Sheet Metal Manufacturing Processes & Materials

Aerospace Fuselage, Ducting & Firewall, Control Panels, and Exhaust

# Manufacturing Processes

Cutting & Blanking

- Laser Cutting (fiber / CO₂) — Precision profile & hole cutting on aluminum, stainless, and titanium sheet; primary process for skin panels and brackets.

- Waterjet Cutting — Cold cutting for heat-sensitive alloys (Ti, Inconel) and thicker firewall/structural blanks where heat-affected zones must be avoided.

- Plasma / Oxy-Fuel Cutting — Heavier-gauge carbon and structural steel blanking; limited aerospace use, mostly tooling and ground-support fixtures.

- Shearing / Nibbling — Straight-line blanking of sheet stock to size ahead of forming.

- Turret / Laser Punching — High-volume hole and cutout patterns in ducting and panel sheet.

Forming & Bending

- Press Brake Forming — Straight and multi-bend forming of brackets, clips, and duct sections.

- Roll Bending / Roll Forming — Continuous curvature for cylindrical and conical duct sections and fuselage skin segments.

- Stamping (progressive die / tandem press) — High-repetition forming of structural ribs, clips, and panel stiffeners.

- Deep Drawing — Dished or domed panels — firewall bulkheads, engine cowling sections.

- Hydroforming / Superplastic Forming — Complex contoured skins in titanium and aluminum where a single-piece compound curve is required.

Joining

- TIG / MIG Welding — Exhaust systems, ducting seams, and firewall assemblies (stainless, Inconel, titanium).

- Riveting — Primary fuselage skin-to-frame and skin-to-stringer joints (solid & blind rivets).

- Resistance Spot Welding — Aluminum panel and bracket assemblies where access allows.

- Brazing — Duct joints and heat-exchanger assemblies in high-temperature zones.

Finishing & Treatment

- Deburring / Edge Break — Standard finishing step on every cut or punched edge before assembly.

- Anodizing / Alodine (chromate conversion) — Corrosion protection for aluminum skins, panels, and brackets.

- Passivation — Corrosion resistance restoration on stainless steel firewall and duct components.

- Heat Treatment (solution + age) — Strength development for 2xxx/7xxx aluminum and precipitation-hardening stainless (17-4PH, 15-5PH).

# Common Materials by Application

| Application | Common Materials |
| --- | --- |
| Fuselage (skin & structure) | Aluminum 2024-T3, 7075-T6, 6061-T6, 2219; Ti-6Al-4V for high-stress frame fittings |
| Ducting & Firewall | Stainless Steel 321, 347, 15-5PH, 17-4PH; Titanium alloys; Inconel 625 in high-heat firewall zones |
| Control Panels | Aluminum 5052, 6061; Stainless Steel 304/316 where corrosion resistance or EMI shielding is required |
| Exhaust Systems | Inconel 625, Inconel 718, Hastelloy X; Titanium Ti-6Al-4V; Stainless Steel 321/347 for oxidation resistance at elevated temperature |

Note: material selection is driven by the specific temperature, corrosion, and load environment of each installation; the above reflects the grades most commonly specified for these application families.